3-mavzu: EtherChannel

Bu mavzu siz Module A'da asoslarini ko‘rgan EtherChannel'dan enterprise darajasida foydalanishga o'tish bo‘ladi.

Nima muammoni hal qiladi?

Tasavvur qiling, ikkita switch orasida bitta link bor:

SW1 ───────── SW2

Bu linkning bandwidth'i, masalan, 100 Mbps bo‘lsa, yana bitta link qo‘shsak:

SW1 ───────── SW2

─────────

oddiy holatda STP ikkinchi linkni bloklashi mumkin.

EtherChannel esa bir nechta fizik linkni bitta mantiqiy link sifatida birlashtiradi:

┌─────────┐

SW1 ======│ Port-   │====== SW2

│Channel  │

└─────────┘

2 × 100 Mbps ≈ 200 Mbps

Shuningdek, bitta fizik link ishdan chiqsa, qolgan linklar ishlashda davom etadi.

Enterprise'da nima uchun kerak?

Masalan, Core va Distribution switchlar orasida:

- Bandwidth oshadi

- Redundancy paydo bo‘ladi

- STP bir nechta linkni alohida ko‘rib bloklash o‘rniga Port-Channel'ni bitta logical link sifatida ko‘radi

- katta tarmoqlarda Core ↔ Distribution ulanishlari uchun juda foydali

Bizning lab

Link 1

SW1 ─────────── SW2

Link 2

SW1 ─────────── SW2

Ikkala linkni EtherChannelga aylantiramiz.

Keyin:

- LACP nima ekanini o‘rganamiz

- SW1 va SW2'ni sozlaymiz

- Port-channelni tekshiramiz

- Trafikni test qilamiz

- Bitta fizik linkni uzib, redundancyni tekshiramiz

- Troubleshooting qilamiz

Muhim: bu safar EtherChannel'ni shunchaki buyruq yodlash uchun emas, nega enterprise tarmoqda kerakligini amalda ko‘ramiz.

Hozir 1-qadam

Packet Tracer'da yangi kichik lab oching:

- 2 ta 2960 switch

- SW1

- SW2

Ularni 2 ta alohida Ethernet kabel bilan ulang.

Masalan:

SW1 Fa0/1 ───────── Fa0/1 SW2

SW1 Fa0/2 ───────── Fa0/2 SW2

2-qadam: LACP

Enterprise tarmoqlarda EtherChannel'ni LACP bilan sozlashni o‘rganamiz.

LACP = Link Aggregation Control Protocol

U ikki switchga:

“Bu fizik linklarni bitta EtherChannel sifatida birlashtiraylik”

deb kelishib ishlash imkonini beradi.

LACP'da asosiy rejimlar:

- active — LACP negotiation boshlaydi

- passive — LACP negotiation'ni kutadi

Biz labda active ↔ active ishlatamiz. Bu orqali ikkala tomon ham negotiation boshlaydi.

3-qadam — SW1 konfiguratsiyasi

SW1'da Fa0/1 va Fa0/2 ni birgalikda tanlaymiz:

Switch> enable

Switch# configure terminal

Switch(config)# interface range FastEthernet0/1 - 2

Switch(config-if-range)# channel-group 1 mode active

Bu yerda:

- interface range → ikkita portni bir vaqtda tanlaydi

- channel-group 1 → ularni Port-Channel 1 ga birlashtiradi

- mode active → LACP negotiation boshlaydi

Keyin:

Switch(config-if-range)# exit

Switch(config)# end

Tekshirish

Switch# show etherchannel summary

Hozir taxminan:

Group  Port-channel  Protocol    Ports

------+-------------+-----------+----------------

1      Po1(SU)       LACP        Fa0/1(P) Fa0/2(P)

kabi chiqishi kerak.

P — port EtherChannel tarkibiga muvaffaqiyatli qo‘shilganini bildiradi.

Belgilar nimani anglatadi?

- S → Layer 2 EtherChannel

- D → Port-channel down

- I → port stand-alone, ya'ni EtherChannel'ga qo‘shilmagan

Demak, SW1 tomoni tayyor, lekin SW2 hali LACP'ga qo‘shilmagan. Shu sababli SW1 linklarni birlashtira olmayapti.

Bu yerda hozircha hech narsani o‘zgartirmaymiz.

4-qadam — SW2'ni sozlaymiz

SW2'da xuddi shu portlarni:

Switch> enable

Switch# configure terminal

Switch(config)# interface range FastEthernet0/1 - 2

Switch(config-if-range)# channel-group 1 mode active

Switch(config-if-range)# exit

Switch(config)# end

Keyin:

Switch# show etherchannel summary

Kutiladigan natija

Endi ikkala port ham P bo‘lishi kerak:

Group  Port-channel  Protocol    Ports

------+-------------+-----------+----------------

1      Po1(SU)       LACP        Fa0/1(P) Fa0/2(P)

Bu yerda:

- SU → Layer 2 + Port-channel ishlayapti

- P → portlar EtherChannel tarkibida

Mana sabab topildi. 👍 Muammo konfiguratsiyada emas, kabel ulanishida.

SW1:

Fa0/1  connected

Fa0/2  connected

SW2:

Fa0/1  connected

Fa0/2  notconnect

Fa0/3  connected

Demak, biz avval aytgan:

SW1 Fa0/1 ↔ SW2 Fa0/1

SW1 Fa0/2 ↔ SW2 Fa0/2

ulanishi amalda bunday emas.

SW2'da ikkinchi kabel Fa0/3 ga ulangan ko‘rinadi.

Hozir nima qilamiz?

Kabelni o‘zgartirish kerak.

Hozirgi holat ehtimol:

SW1 Fa0/1 ───────── SW2 Fa0/1

SW1 Fa0/2 ───────── SW2 Fa0/3

Biz esa quyidagisini xohlaymiz:

SW1 Fa0/1 ───────── SW2 Fa0/1

SW1 Fa0/2 ───────── SW2 Fa0/2

Shuning uchun SW2 tomonidagi ikkinchi kabelni Fa0/3 dan uzib, Fa0/2 ga ulang.

Birinchi kabelga tegmang.

Ulab bo‘lgach, SW2'da:

Switch# show interfaces status

qiling.

Fa0/1 ham, Fa0/2 ham:

connected

bo‘lishi kerak.

Keyin show etherchannel summaryni tekshiramiz.

Switch#show interfaces status Port Name Status Vlan Duplex Speed Type Fa0/1 connected 1 a-full a-100 10/100BaseTX Fa0/2 connected 1 a-full a-100 10/100BaseTX Fa0/3 notconnect 1 auto auto 10/100BaseTX Fa0/4 notconnect 1 auto auto 10/100BaseTX Fa0/5 notconnect 1 auto auto 10/100BaseTX Fa0/6 notconnect 1 auto auto 10/100BaseTX Fa0/7 notconnect 1 auto auto 10/100BaseTX Fa0/8 notconnect 1 auto auto 10/100BaseTX Fa0/9 notconnect 1 auto auto 10/100BaseTX Fa0/10 notconnect 1 auto auto 10/100BaseTX Fa0/11 notconnect 1 auto auto 10/100BaseTX Fa0/12 notconnect 1 auto auto 10/100BaseTX Fa0/13 notconnect 1 auto auto 10/100BaseTX Fa0/14 notconnect 1 auto auto 10/100BaseTX Fa0/15 notconnect 1 auto auto 10/100BaseTX Fa0/16 notconnect 1 auto auto 10/100BaseTX Fa0/17 notconnect 1 auto auto 10/100BaseTX Fa0/18 notconnect 1 auto auto 10/100BaseTX Fa0/19 notconnect 1 auto auto 10/100BaseTX Fa0/20 notconnect 1 auto auto 10/100BaseTX Fa0/21 notconnect 1 auto auto 10/100BaseTX Fa0/22 notconnect 1 auto auto 10/100BaseTX Fa0/23 notconnect 1 auto auto 10/100BaseTX Fa0/24 notconnect 1 auto auto 10/100BaseTX

Zo‘r, endi fizik ulanish to‘g‘ri. ✅

SW1 Fa0/1 ↔ SW2 Fa0/1

SW1 Fa0/2 ↔ SW2 Fa0/2

Ikkalasi ham connected.

Endi LACP negotiation ishlashi kerak.

Keyingi tekshiruv

SW1'da:

Switch# show etherchannel summary

SW2'da ham:

Switch# show etherchannel summary

Kutiladigan holat:

Po1(SU)

Fa0/1(P) Fa0/2(P)

Agar ikkala switchda ham P chiqsa, EtherChannel to‘liq ishlayapti.

Ikkala natijani yuboring.

SW 2 Switch#show etherchannel summary Flags: D - down P - in port-channel I - stand-alone s - suspended H - Hot-standby (LACP only) R - Layer3 S - Layer2 U - in use f - failed to allocate aggregator u - unsuitable for bundling w - waiting to be aggregated d - default port Number of channel-groups in use: 0 Number of aggregators: 0 Group Port-channel Protocol Ports ------+-------------+-----------+---------------------------------------------- SW1 Switch#show etherchannel summary Flags: D - down P - in port-channel I - stand-alone s - suspended H - Hot-standby (LACP only) R - Layer3 S - Layer2 U - in use f - failed to allocate aggregator u - unsuitable for bundling w - waiting to be aggregated d - default port Number of channel-groups in use: 1 Number of aggregators: 1 Group Port-channel Protocol Ports ------+-------------+-----------+---------------------------------------------- 1 Po1(SD) LACP Fa0/1(I) Fa0/2(I)

Ha, endi muammo aniq ko‘rinyapti. Fizik ulanish to‘g‘ri, lekin SW2 da EtherChannel umuman konfiguratsiya qilinmagan.

SW2:

Number of channel-groups in use: 0

SW1:

1  Po1(SD)  LACP  Fa0/1(I) Fa0/2(I)

Demak, hozir SW1 LACP kutyapti, SW2 esa hali LACP guruhiga qo‘shilmagan.

1-qadam — faqat SW2 ni sozlaymiz

SW2 da:

Switch> enable

Switch# configure terminal

Switch(config)# interface range FastEthernet0/1 - 2

Switch(config-if-range)# channel-group 1 mode active

Switch(config-if-range)# end

Bu yerda:

- interface range Fa0/1 - 2 → ikkala fizik portni tanlaydi.

- channel-group 1 → ikkala portni Port-Channel 1 ga birlashtiradi.

- mode active → LACP muzokarasini faol boshlaydi.

2-qadam — tekshiramiz

Hozircha boshqa hech narsa qilmaymiz.

SW2 da:

Switch# show etherchannel summary

SW1 da ham:

Switch# show etherchannel summary

natija kutilganidek buldi